Stormwater Pond Inspections and Maintenance Checklist

Site Name: 	Owner Change since last inspection? Y N

Location:

Owner Name:

Address: 	Phone Number:

Site Status:

Date: 	Time: 	Site conditions:

Stormwater Pond Type:	Wet Pond		Wet ED Pond  	Micropool Pond	  Multiple Pond System	

Dry Pond 

Inspection Frequency Key: A=annual (required); M=monthly (recommended); S=after major storms (recommended)

| Inspection Items | Inspection Frequency | Inspected? (Yes/No) | Maintenance Needed? (Yes/No) | Comments/Description |
| --- | --- | --- | --- | --- |
| Embankment and Emergency Spillway |  |  |  |  |
| Vegetation healthy? | A/S |  |  |  |
| Erosion on embankment? | A/S |  |  |  |
| Animal burrows in embankment? | A/S |  |  |  |
| Cracking, sliding, bulging of dam? | A/S |  |  |  |
| Drains blocked or not functioning? | A/S |  |  |  |
| Leaks or seeps on embankment? | A/S |  |  |  |
| Slope protection failure functional? | A/S |  |  |  |
| Emergency spillway obstructed? | A/S |  |  |  |
| Erosion in/around emergency spillway? | A/S |  |  |  |
| Other (describe) | A/S |  |  |  |
| Riser and Principal Spillway |  |  |  | (describe type: concrete pipe, slotted weir, channel, etc.) |
| Low-flow orifice functional? | A/S |  |  |  |
| Trash rack (Debris removal needed? Corrosion noted?) | A/S |  |  |  |
| Sediment buildup in riser? | A |  |  |  |
| Concrete/masonry condition (Cracks or displacement? Spalling?) | A |  |  |  |
| Metal pipe in good condition? | A |  |  |  |
| Control valve operation? | A |  |  |  |
| Pond drain valve operation? | A |  |  |  |
| Outfall channels function, not eroding? | A |  |  |  |
| Other (describe) | A |  |  |  |
| Sediment Forebays |  |  |  |  |
| Sedimentation description |  |  |  |  |
| Sediment cleanout needed (over 50 | A/S |  |  |  |

| Inspection Items | Inspection Frequency | Inspected? (Yes/No) | Maintenance Needed? (Yes/No) | Comments/Description |
| --- | --- | --- | --- | --- |
| percent full)? |  |  |  |  |
| Permanent Pool Areas (if applicable) |  |  |  |  |
| Undesirable vegetation growth? | A/M |  |  |  |
| Visible pollution? | A/M |  |  |  |
| Shoreline erosion? | A/M |  |  |  |
| Erosion at outfalls into pond? | A/M |  |  |  |
| Headwalls and endwalls in good condition? | A/M |  |  |  |
| Encroachment into pond or easement area by other activities? | A/M |  |  |  |
| Evidence of sediment accumulation? | A |  |  |  |
| Dry Pond Areas (if applicable) |  |  |  |  |
| Vegetation adequate? | A/M |  |  |  |
| Undesirable vegetation or woody plant growth? | A/M |  |  |  |
| Excessive sedimentation? | A |  |  |  |
| Hazards |  |  |  |  |
| Have there been complaints from residents? | A/M |  |  |  |
| Public hazards noted? | A/M |  |  |  |

Inspector Comments:

Overall Condition of Facility:    Acceptable	 Unacceptable

If any of the above Inspection Items are checked “Yes” for “Maintenance Needed,” list Maintenance actions and their completion dates below:

| Maintenance Action Needed | Due Date |
| --- | --- |

The next routine inspection is scheduled for approximately:

(date) Inspected by: (signature)  		 Inspected by: (printed)