Grass Channel Inspections and Maintenance Checklist

Site Name: 	Owner Change since last inspection? Y N

Location:

Owner Name:

Address: 	Phone Number:

Site Status:

Date: 	Time: 	Site conditions:

Inspection Frequency Key: A=annual; M=monthly; S=after major storms

| Inspection Items | Inspection Frequency | Inspected? (Yes/No) | Maintenance Needed? (Yes/No) | Comments/Description |
| --- | --- | --- | --- | --- |
| Debris Removal |  |  |  |  |
| Facility and adjacent area free of debris? | M |  |  |  |
| Inlets and outlets free of debris? | M |  |  |  |
| Any dumping of yard wastes into facility? | M |  |  |  |
| Litter (branches) removed? | M |  |  |  |
| Vegetation |  |  |  |  |
| Surrounding area fully stabilized? (no evidence of eroding material into swale, channel or filter strip) | M |  |  |  |
| Grass mowed? | M |  |  |  |
| Grass height not less than 3 to 4 inches? | M |  |  |  |
| Fertilized per specifications? | M |  |  |  |
| Grasses planted according to approved plan? | M |  |  |  |
| Unauthorized or inappropriate plantings? | A |  |  |  |
| Grasses healthy? (no diseased or dying vegetation) | M |  |  |  |
| Evidence of grasses stressed from inadequate watering? | M |  |  |  |
| Filtration Capacity |  |  |  |  |
| Clogging from oil or grease? | M |  |  |  |
| Facility dewaters between storms? | M |  |  |  |
| Check dams and energy dissipaters/sumps |  |  |  |  |
| Any evidence of sedimentation buildup? | A,S |  |  |  |
| Are sumps greater than 50% full of sediment? | A,S |  |  |  |

| Inspection Items | Inspection Frequency | Inspected? (Yes/No) | Maintenance Needed? (Yes/No) | Comments/Description |
| --- | --- | --- | --- | --- |
| Any evidence of erosion and downstream toe of drop structures? | A,S |  |  |  |
| Any trash or blockages at weep holes? | A,S |  |  |  |
| Sediment Deposition |  |  |  |  |
| Swale clean of sediments? | A |  |  |  |
| Sediment not > 25% of swale design depth? | A |  |  |  |
| Outlet/Overflow Spillway |  |  |  |  |
| In good condition? | A |  |  |  |
| Any evidence of erosion? | A |  |  |  |
| Any evidence of blockages? | A |  |  |  |
| Has facility been filled or blocked inappropriately? | A |  |  |  |
| Hazards |  |  |  |  |
| Have there been complaints from residents? | M |  |  |  |
| Public hazards noted? | M |  |  |  |
| Maintenance accesses free of hazards and fully operational? | M |  |  |  |

Inspector Comments:

Overall Condition of Facility:    Acceptable	 Unacceptable

If any of the above Inspection Items are checked “Yes” for “Maintenance Needed,” list Maintenance actions and their completion dates below:

| Maintenance Action Needed | Due Date |
| --- | --- |

The next routine inspection is scheduled for approximately:

(date) Inspected by: (signature)  		 Inspected by: (printed)